Mit Field App Workflows

Mit Tech

Links to Process Steps:

These six (6) steps align with the standard “Job” steps in the app

At the Office

INSTALL, DEMO, CHECK SERVICE, PULL

At the Property

INSTALL, DEMO, CHECK SERVICE, PULL

At the Front Door

INSTALL, DEMO, CHECK SERVICE, PULL

Inside the House

INSTALL

DEMO

CHECK SERVICE

PULL

Leaving the House

INSTALL, DEMO, CHECK SERVICE, PULL

Back at the Truck

INSTALL, DEMO, CHECK SERVICE, PULL

# At the Office

## INSTALL, DEMO, CHECK SERVICE, PULL

### RAJT

- Review the Jobs on your list

### Truck Prep

- Ensure there is “A Place for Everything and Everything is in its Place”

- Gather the equipment and tools needed for the scheduled jobs

# At the Property

## INSTALL, DEMO, CHECK SERVICE, PULL

### Property Arrival

- Park safely, set up safety cones, take photo of truck location and property exterior

### Clock In

- Log arrival time in app

### Review Job Details

- Confirm job information, insurance carrier, special requirements, and DC notes

# At the Front Door

## INSTALL, DEMO, CHECK SERVICE, PULL

### Customer Introduction

- Present Entrusted identification, introduce yourself and team

### Ground Rules Presentation

- Explain purpose of visit, today's process, estimated timeline, noise levels, access needs

- Ground Rules 5 Steps

  - 1. Build Rapport

  - 2. Lay the Ground Rules for today's meeting (see document)

  - 3. Do Assessment

  - 4. Discover/Uncover pain

  - 5. Offer Solutions/Go Over Roadmap

### Property Walkthrough with Customer

- Tour affected areas, listen to customer's description of water event, note customer concerns, identify valuables needing protection, confirm utility locations

### Address Customer Questions

- Answer immediate concerns, establish communication preferences

# Inside the House

## INSTALL

### Identify Pre-existing Conditions

- Anything of concern not related to the loss

- Damage, Pet Odor, etc.

### Start at Cause of Loss

- Identify and photograph the cause of loss

- Document category of water (Category 1, 2, or 3)

### Room by Room Evaluation

- Work outward from cause of loss to find edge of damage, going room by room

- Follow the “Don’t trust water steps”

- Complete all rooms before moving on

### Affected Rooms

- For each room: name it, take overall photos, capture measurements, take thermal imaging and moisture meter readings of affected and unaffected materials

### Unaffected Rooms

- For each room: name it, take overall photos, capture measurements, take thermal imaging and moisture meter readings of affected and unaffected materials

- Document unaffected rooms that can't be closed off from drying chambers with measurements

### Mitigate and Install (this step can be done at any point during the install)

- Light demo (i.e. remove toe kicks, baseboards, carpet pad, etc)

- Equipment Placement

  - For each piece of equipment (starting with dehumidifiers)

  - Room by room and chamber by chamber, place the equipment based on the calculations

### Define Drying Chambers

- Determine number of chambers and assign rooms to each chamber

### Equipment Calculations

- System calculates recommended equipment based on measurements and chamber configuration per IICRC Standard

### Develop & Communicate the Plan (Post Inspection Conversation)

- Give customer an update on what you found and additional work needed

- If needed, discuss and perform “install demo” (i.e. toe kicks, baseboards, carpet pad)

- Develop final plan

  - Ensures customer buy-in before equipment deployment

### Final Documentation & Depart

- Equipment Scanning

  - Scan equipment

  - Assign room (and chamber)

  - Safety Prompts

  - Take photo

- Job Plan: Create the plan for subsequent visits (demo schedule, check services, etc.

- Complete notes, ensure all required fields are filled

## DEMO

### Pre-Demo Safety Setup

- Set up safety equipment, PPE, containment, floor protection, and safety cones

### Pre-Demo Documentation & Verification

- Take before photos of all areas to be demoed

- Verify conditions before demo

### Room-by-Room Demo

- For each room: demo affected materials (drywall, flooring, baseboards, etc.), document square footage removed, take photos of newly exposed areas, assess for additional hidden damage, disinfect exposed framing/surfaces

### Disposal Documentation

- Document disposal method and debris removal

### Post-Demo Assessment

- Inspect newly exposed areas and take moisture readings on newly exposed materials

### Equipment Adjustment

- Add, move, or adjust equipment based on post-demo conditions

### Post-Demo Documentation

- Take after photos showing completed demo work

### Site Cleanup

- Remove debris, clean work area, ensure safe conditions

### Job Planning Update

- Update plan for next visit based on post-demo conditions

### Equipment Scanning (Room by Room)

- Scan the Dehus only (not Air Scrubbers or Air Movers) in each room and take equipment photos including readings photos (for dehumidifiers after 20-30 min of run time)

- Take final overall photos of the job

### Final Documentation

- Complete notes, ensure all required fields are filled

## CHECK SERVICE

### Chamber Readings

- Take temperature and humidity readings for each drying chamber

### Room-by-Room Assessment

- For each room: check moisture levels on all monitored materials, inspect equipment function and position, adjust/move/add/remove equipment as needed, take photos showing drying progress

- Record temperature, RH, and hours running for each dehumidifier in the chamber

- Pull or add any equipment as needed

- Take equipment photos of all areas

### Progress Evaluation

- Review overall drying progress across all rooms and chambers

### Final Documentation & Depart

- Equipment Scanning

  - Scan equipment

  - Assign room (and chamber)

  - Safety Prompts

  - Take photo

- Job Plan: Create the plan for subsequent visits (demo schedule, check services, etc.

- Complete notes, ensure all required fields are filled

## PULL

### Final Moisture Readings

- Take final moisture readings on all monitored materials throughout the property to confirm drying is complete

### Room-by-Room Equipment Removal

- For each room: verify materials are dry and remove all equipment (dehumidifiers, air movers, air scrubbers), take final photos of dried areas (Note: Equipment will be scanned AT THE TRUCK, not IN THE HOME)

  - Need the ability to complete a partial pull

  - Scan all equipment in the home or at the truck

### Equipment Inspection

- Check all removed equipment for damage and proper function before loading

### Site Restoration

- Remove containment, clean work area, leave jobsite broom swept/vacuumed

# Leaving the House

## INSTALL, DEMO, CHECK SERVICE, PULL

### Final Documentation Review

- Verify all photos uploaded and organized by room

- Confirm all materials, readings, and notes are complete

- Red flag any missing information that must be addressed

### Customer Final Walkthrough

- Walk customer through all work completed today

- Explain what was done and what comes next

- Address any customer questions or concerns

- Explain next service date and what to expect

- Provide POC information for questions

- Update any changes to access instructions or customer contact info

### Payment Collection (cash jobs only)

- Verify payment amount due (highlights in app)

- Collect second half of payment if applicable

- Upload proof of payment

### Matterport Completion (verify done)

- Confirm Matterport scan has been completed

- Upload Matterport documentation if required

- Check off completion in app

### Dry Release Waiver (if applicable)

- Verify if DRW is needed (past contract date)

- Ensure homeowner was prepped for DRW by Mit Lead

- Obtain signature and upload waiver

# Back at the Truck

## INSTALL, DEMO, CHECK SERVICE, PULL

### Load Equipment Back in Truck

- Secure all equipment and tools used at this job

- Organize equipment properly in truck storage

- Ensure nothing is left at customer property

### Verify Job Completion in App

- Confirm all required tasks marked complete for this job

- Check for any red flags or missing information

- Review next job on schedule

### Call Next Customer

- Contact next customer to confirm ETA

- Review job details and special instructions for next stop

- Verify access instructions and customer availability

### Drive to Next Job

- Navigate to next customer location

- Prepare mentally for next service type (install/demo/check/pull)

- Use travel time to review next job information in app